# Mathematics of the Satellite RF Interferometer

October 18, 2026

Table of Contents

This note records the mathematics implemented in the Satellite RF Interferometer Lab (version 0.3.0). The lab has two layers. The array layer predicts the phase a narrowband interferometer would measure from a far-field direction, and it searches those phases for the directions that could have produced them. The satellite layer places that array on a low-Earth-orbit spacecraft, turns a ground emitter into a body-frame direction, and turns a candidate direction back into a latitude and longitude on the WGS84 ellipsoid.

The equations below are the ones the program evaluates. The numerical example at the end uses the default values shown in the app.

# 1	What is being estimated

A transmitter at a known radio frequency illuminates a small antenna array. Each antenna samples the same narrowband signal. Because the array is tiny compared with the range, the wave is a plane wave, and the only geometric information in the carrier phase is the direction of arrival.

The observable used everywhere in the lab is the phase of antenna j minus the phase of antenna 0. Those phases are then reduced to a principal value in [−π,π). From a set of wrapped phases the program searches the front hemisphere for directions that reproduce them, refines each basin, and, on the satellite tab, extends each direction until it meets the Earth.

The estimator that reports a latitude and longitude receives only the wrapped phases, the array geometry, the frequency, the satellite position, and the attitude it is told to use. It does not receive the true emitter coordinates. Those coordinates are used only to draw the truth and to score the error after the fact.

# 2	Modeling assumptions

The phase model is far-field, narrowband, and simultaneous. The elements are ideal and isotropic. The complex phase at a point r is

exp​+j 2πλ r⋅u,

where u points from the array toward the source. With the e+jωt time convention, a propagation delay is a negative phase. An element displaced toward the source has a shorter path and therefore a phase lead, which is what the dot product above produces.

The following effects are left out of the model: thermal-noise conversion from signal-to-noise ratio into phase variance, multipath, polarization, mutual coupling, element patterns, receiver delay calibration beyond a single phase bias, Earth rotation during the snapshot, and orbit propagation. The satellite velocity direction is the heading of a spherical circular orbit at the spacecraft’s current geocentric latitude. The position itself sits on the WGS84 ellipsoid at a chosen height. The program does not integrate a trajectory.

# 3	Constants and symbols

| Symbol | Meaning |
| --- | --- |
| c=299 792 458 m s−1 | Speed of light |
| f | Carrier frequency |
| λ=c/f | Wavelength |
| α | Azimuth, counterclockwise from the antenna +X axis |
| θ | Off-boresight angle, measured from the antenna +Z axis |
| u | Unit vector from the satellite (or the array) toward the emitter |
| rj | Position of antenna j; antenna 0 is the phase reference |
| φ, λ | Geodetic latitude and longitude. Longitude is east-positive. In the radio-frequency sections, λ is the wavelength; the two uses do not appear in the same equation |
| h | Height above the WGS84 ellipsoid |
| δ | Geocentric latitude |
| i | Orbit inclination, from 0∘ to 180∘ |
| u | Argument of latitude, measured from the ascending node |
| ρ, μ, ψ | Body roll, pitch, and yaw relative to the orbital frame |

WGS84 is the ellipsoid used for every ground coordinate:

a&=6 378 137 m,f&=1/298.257 223 563,b&=a(1−f),e2&=f(2−f),e′​2&=e2/(1−e2).

Numerically, b=6 356 752.314 m, e2=0.006 694 380, and e′​2=0.006 739 497.

# 4	The interferometer

## 4.1	Wavelength

λ=cf.

Frequency in the lab runs from 0.1 to 40 GHz. At the default 2.2 GHz,

λ=0.136269 m=13.627 cm.

## 4.2	Arrival direction

Antennas lie in the XY plane. The +Z axis is the boresight, and the front hemisphere is 0∘≤θ≤90∘. Azimuth α is measured counterclockwise from +X. The unit arrival vector is

u=sinθcosαsinθsinαcosθ.

The same direction is often written by its direction cosines

ux=sinθcosα,  uy=sinθsinα,  uz=cosθ,

so that ux2+uy2+uz2=1. On a planar array the carrier phase depends only on ux and uy. The sign of uz is invisible to the phase, which is why the search explicitly assumes the front hemisphere, uz=+1−ux2−uy2.

Given a unit vector, the angles are recovered by

θ=atan2​ux2+uy2, uz,  α=atan2(uy,ux) mod 360∘.

At boresight, ux=uy=0 and the azimuth is undefined. The off-boresight angle is zero only on the +Z axis.

## 4.3	Array layouts

Antenna 0 is at the origin and is the phase reference. Every measurement is a baseline from antenna 0 to antenna j. Baselines between the other elements are not formed. All elements have z=0.

Let S be the short baseline and L the long baseline, both in meters. The six layouts are

| Layout | Antenna coordinates (x,y), in meters |
| --- | --- |
| 2-element | (0,0), (L,0) |
| 3-linear | (0,0), (S,0), (L,0) |
| 3-L | (0,0), (L,0), (0,L) |
| 3-triangle | (0,0), (L,0), L/2, (3/2) L |
| 4-dual-axis | (0,0), (S,0), (0,S), (L,L) |
| 5-cross | (0,0), (S,0), (0,S), (L,0), (0,L) |

The triangle is equilateral with side L. The dual-axis element at (L,L) lies 2 L from the origin.

The horizontal baseline matrix is the stack of vectors (rj−r0) with the z component dropped. Its rank is the number of independent direction cosines the geometry can observe:

- a two-element or linear array has rank 1, so only one direction cosine is constrained;

- an L, a triangle, a dual-axis array, or a cross has rank 2, so both ux and uy enter the phase.

Rank 2 removes the collinear ambiguity. It does not remove the wrapping ambiguity discussed below. A long baseline of rank 2 can still be satisfied by many directions.

## 4.4	Phase of a reference pair

The extra geometric path length at antenna j, relative to antenna 0, for a source in direction u is

Δℓ0j=−(rj−r0)⋅u.

The minus sign says the path is shorter when the element is displaced toward the source. Under the e+jωt convention the corresponding phase lead, in radians, is

Δϕ0j=2πλ (rj−r0)⋅u=k (rj−r0)⋅u,  k=2πλ.

This is the unwrapped phase. It is proportional to the projected baseline. In the principal plane used by the phase plot, azimuth is fixed at 0∘ and the angle γ runs from −90∘ to +90∘, from −X through +Z to +X. A baseline of length bx along X then has

Δϕ=2πλ bxsinγ.

The slope dϕ/dγ=(2πbx/λ)cosγ is steepest at boresight and zero at endfire. A longer baseline is locally more sensitive, and it completes more phase cycles as γ sweeps the hemisphere.

## 4.5	Wrapping

A receiver reports the principal value

wrap(ϕ)=(ϕ+π) mod 2π − π,

which lies in [−π,π). Two unwrapped phases that differ by an integer number of cycles are indistinguishable after this reduction.

On one baseline b=rj−r0, two arrival directions u and u′ produce the same wrapped phase when

b⋅(u−u′)=nλ,  n∈Z.

Along a single axis the front hemisphere spans a direction cosine from −1 to +1, so the unwrapped phase spans

± 2πBλ

for a baseline of length B. That span stays inside (−π,π) when

B<λ2.

A baseline shorter than half a wavelength therefore does not wrap anywhere in the front hemisphere. At the default frequency, half a wavelength is 0.06813 m. The stock long baseline of 0.8 m is 5.87 λ, so each arm of the default L array wraps several times.

## 4.6	What the program treats as a measurement

The measured phase on each reference pair is the wrapped sum of the true phase, a bias, and a noise draw:

ϕmeas=wrap​Δϕ+b+ε,  ε∼N(0,σ2).

The draws on different pairs are independent. There is no signal-to-noise formula behind σ; the standard deviation is an angle entered directly, in degrees, and converted to radians before it is added.

Two bias models appear in the lab.

- On the array tabs, and as an extra offset on the satellite tab, one common angle b is added to every reference pair.

- When channel calibration errors are enabled, each antenna has its own phase cj and the pair bias is the difference that a shared reference produces,

bj=cj−c0.

In the Monte Carlo run those channel phases are drawn once, cj∼N(0,σc2), and then held fixed while the random phase noise changes from trial to trial.

# 5	From phases back to a direction

The inverse problem asks for every front-hemisphere direction whose predicted wrapped phases agree with the measurement. Because of wrapping, the cost surface has many basins when any baseline is longer than about λ/2.

## 5.1	Search grid

The search coordinates are the direction cosines (ux,uy), not azimuth and elevation. For a requested angular step Δθ (the sidebar choices are 4∘, 2∘, 1∘, and 0.5∘) each axis is sampled by

N=round​2sinΔθ+1

points spaced uniformly from −1 to 1. The spacing is Δu=2/(N−1)≈sinΔθ. Near boresight, where u≈θ in radians, that spacing is about Δθ of angle. Farther off boresight the same Δu covers a larger angle, because du=cosθ dθ.

A grid point is kept only when it lies in the unit disk,

ux2+uy2≤1,

which is the front hemisphere. The outer boundary of the disk is the antenna horizon, θ=90∘.

At the default 2∘ step the axis has 58 samples and Δu=0.03509.

## 5.2	Mismatch cost

On baseline j, with horizontal components (bx,j,by,j), the phase predicted by a trial direction cosine is

ϕj=k (bx,j ux+by,j uy).

The wrapped residual is δj=wrap(ϕj−ϕmeas,j). Subtracting the raw angles would jump by 2π every time a residual crossed the branch cut, so the cost uses the chord length between the two unit phasors ejϕ and ejϕmeas:

ejδ−1=2sin(δ/2).

With M reference pairs the cost plotted on the ambiguity map is the root-mean-square chord

s(ux,uy)=1Mj=1M2sinδj22.

The quantity s is dimensionless and lies between 0 and 2. The heatmap shows s. A table that quotes a phase mismatch in degrees converts that score back to an equivalent angle,

ρ=2arcsin​clip(s/2, 0, 1).

If every pair has the same residual δ, then ρ=|δ|. If the pair residuals differ, ρ is the single angle whose chord equals the RMS chord. It is a summary of the phase fit. It is not a formal uncertainty.

## 5.3	Local minima on the grid

A grid point is a candidate when its cost is less than or equal to the cost of its eight neighbors. Neighbors that fall outside the disk are ignored. The minima are sorted from lowest cost to highest. A minimum is kept when its distance in the (ux,uy) plane from every minimum already kept is at least

d=max​0.035, 2sinΔθ.

At most 12 minima are retained. Their angles are

α=atan2(uy,ux) mod 360∘,  θ=arcsin​min(1,ux2+uy2).

For uz≥0 this θ matches the atan2 formula above. The cap of 12 is a listing limit. A long baseline can have more aliases than the list contains, and a basin that falls between grid points is not refined later.

## 5.4	Continuous refinement

Each retained grid minimum is the seed of a small nonlinear least-squares problem in the two direction cosines w=(ux,uy). The residual vector stacks one circular phase error per pair and a penalty that turns on outside the unit disk:

r(w)=2sin​wrap(k Bw−ϕmeas)/25max(0, ∥w∥2−1).

Here B is the M×2 matrix of horizontal baselines. The fit is a bounded trust-region least squares with each component of w confined to [−1,1]. The true direction is not provided to the fit.

If the converged w lies outside the unit disk, it is projected back to the circle and uz is set to 0. Otherwise

uz=1−ux2−uy2.

After refinement, directions separated by less than 0.4∘ are treated as the same basin and only the better one is kept. The surviving directions are sorted by the equivalent phase angle ρ.

# 6	The satellite geometry

On the geolocation tab the same phase model is used without change. The new work is the chain of frames that turns an emitter on the Earth into an antenna-frame direction, and the reverse chain that turns a direction into a ground point.

## 6.1	Frames

Five frames are used.

ECEF. Earth-centered, Earth-fixed. +X pierces the equator at longitude 0, +Y pierces the equator at longitude 90∘E, and +Z pierces the north pole. Coordinates are meters.

Geodetic. Latitude φ, longitude λ, and ellipsoidal height h on WGS84. The outward geodetic normal at (φ,λ) is

n=cosφcosλcosφsinλsinφ.

The satellite subpoint drawn on the globe is the ellipsoid point with the satellite’s geodetic latitude and longitude and with h=0. That point is the foot of the geodetic normal. It is the place a vertical line from the satellite meets the ellipsoid only when the normal and the geocentric radius coincide, which is on the equator and at the poles.

VVLH. The local orbital frame used here is right-handed and nadir-positive:

+X& along-track, the circular-orbit velocity,+Y& cross-track,+Z& geocentric nadir, −r.

The cross-track axis satisfies y=z×x=−h, where h is the direction of the orbit’s angular momentum. This +Z axis points at the center of the Earth. It is the geocentric radius, with a minus sign. It is a different direction from the geodetic normal n once the satellite leaves the equator.

Body. Coincident with VVLH when roll, pitch, and yaw are all zero.

Antenna. Coincident with the body. The interferometer +Z boresight is the body +Z axis, so a nadir-pointing satellite looks at geocentric nadir and the Earth fills the front hemisphere out to the horizon.

## 6.2	Geodetic and ECEF coordinates

The prime-vertical radius of curvature is

N(φ)=a1−e2sin2φ.

A geodetic point becomes ECEF by

X&=(N+h)cosφcosλ,Y&=(N+h)cosφsinλ,Z&=N(1−e2)+hsinφ.

The inverse, ECEF back to geodetic, follows Bowring’s reduced-latitude formula and then updates the latitude twice. Write p=X2+Y2 and λ=atan2(Y,X). The auxiliary angle is

ϑ=atan2(Za, pb),

and the starting latitude is

φ=atan2​Z+e′​2bsin3ϑ, p−e2acos3ϑ.

Each of the two updates replaces the height and the latitude by

N&=a1−e2sin2φ,h&=pcosφ−N,φ&=atan2​Z, p1−e2 NN+h.

Height is then recomputed from the finished latitude so that (X,Y,Z) and (φ,λ,h) agree. On the polar axis, p<10−9 m, the latitude is sign(Z)⋅90∘ and the height is |Z|−b.

Ground distance between two ellipsoid points is the WGS84 geodesic, evaluated by PROJ through pyproj (Karney’s algorithm). That distance is what the lab reports as the geolocation error. It is a distance along the ellipsoid, not a straight chord through the Earth.

## 6.3	Building the orbital frame

The satellite position is the WGS84 point at the chosen geodetic subpoint and altitude. From that ECEF vector r,

r=r/∥r∥,  δ=atan2​Z, X2+Y2,  λ=atan2(Y,X).

Here δ and λ are geocentric. They differ from the geodetic latitude and longitude that were used to build r. Longitude is the same for a point on the geodetic normal; latitude is not.

Inclination i is the angle between the orbit normal and ECEF +Z. For a unit radial vector and a unit velocity perpendicular to it, h=r×v and h⋅Z=cosi. The same triple product is v⋅(Z×r), and Z×r=cosδ e. The east component of the horizontal unit velocity is therefore cosi/cosδ, and the north component is whatever remains on the unit circle:

vE=cosicosδ,  vN=±1−vE2.

The north sign is positive when the pass is ascending, which means geocentric latitude is increasing. If |vE|>1, the requested latitude is unreachable on that orbit. The largest absolute geocentric latitude a spherical circular orbit can reach is

|δ|max=arccos​|cosi|=min(i, 180∘−i).

The local east and north unit vectors at geocentric (δ,λ) are

e&=(−sinλ, cosλ, 0),n​EN&=(−sinδcosλ, −sinδsinλ, cosδ).

The horizontal heading is assembled and then stripped of any leftover radial component so that it is exactly perpendicular to r:

v=vh−(vh⋅r) r∥vh−(vh⋅r) r∥,  vh=vE e+vN n​EN.

The VVLH basis, expressed in ECEF, is then

z=−r,  y=z×v∥z×v∥,  x=y×z.

The matrix RECEF←VVLH has these three unit vectors as its columns. Its transpose maps ECEF components into VVLH.

The ground-track heading, measured from north toward east, is

H=atan2(vE,vN).

The argument of latitude follows from sinδ=sinisinu:

sinu=sinδsini,  cosu=±1−sin2u,  u=atan2(sinu,cosu).

The cosine is positive on an ascending pass. That is the half of the orbit on which latitude is increasing, because d(sinδ)/du=sinicosu. For an equatorial orbit, |sini| is effectively zero and u is left undefined. A subpoint by itself does not determine H or u; the inclination and the ascending or descending flag do.

The construction needs a usable east-north basis, so a satellite within about 10−8 radians of a pole is rejected. Altitude must be positive and inclination must lie in [0∘,180∘].

## 6.4	Attitude

Attitude is an intrinsic yaw, then pitch, then roll. Positive angles follow the right-hand rule about the body axes:

- positive yaw rotates the nose, body +X, toward body +Y (nose to the right, with +Z down);

- positive pitch rotates the nose toward body −Z (nose up, away from the Earth);

- positive roll rotates the right wing, body +Y, toward body +Z (right wing down).

The elementary rotations are

Rx(ρ)&=1000cosρ−sinρ0sinρcosρ,Ry(μ)&=cosμ0sinμ010−sinμ0cosμ,Rz(ψ)&=cosψ−sinψ0sinψcosψ0001.

A vector whose components are known in the body frame is mapped into VVLH by

vVVLH=Rz(ψ) Ry(μ) Rx(ρ) vbody,

and from there into ECEF by RECEF←VVLH. The reverse map is the transpose of each factor, applied in the reverse order. At zero attitude the two frames coincide, body +Z is geocentric nadir, and the body off-nadir angle equals the angle between the line of sight and −r.

## 6.5	Line of sight

An emitter is a point on the ellipsoid, h=0. It may be given directly as a latitude and longitude, or as the ground intercept of a chosen body azimuth and off-nadir angle.

For an emitter at re the line of sight and the slant range are

s=re−rsat,  R=∥s∥,  uECEF=s/R.

The range must be at least one meter. Rotating uECEF into the body frame and reading off α and θ produces the angles that enter the phase equation. The geometric off-nadir angle reported beside them is the angle between uECEF and geocentric nadir −r,

θgeo=arccos​clip(uECEF⋅(−r), −1, 1).

The elevation of the satellite as seen from the ground point, used by the sensitivity curves, is the angle between the line of sight and the local horizontal:

sinε=clip​n⋅(rsat−re)R, −1, 1.

## 6.6	Horizon

The emitter is above the geometric horizon when the satellite lies on the outer side of the local tangent plane. With a 10 m tolerance,

n⋅(rsat−re)≥−10 m.

A point that fails this test is drawn and labeled, and no phase is formed for it: the Earth is in the way. A point that is above the horizon but behind the antenna, θ>90∘, is also left uninverted. The front-hemisphere test is θ≤90∘.

The horizon angle drawn on the globe, at a given antenna azimuth, is the largest off-boresight angle in [0∘,90∘] whose ray still meets the ellipsoid. It is found by bisection. If the boresight itself misses the Earth, that azimuth has no horizon. If a ray at 90∘ still hits, the horizon is 90∘.

## 6.7	Where a direction meets the Earth

A candidate direction u in the body frame is rotated to a unit vector d in ECEF. The ray

p(t)=rsat+t d,  t>0,

is intersected with the ellipsoid

x2+y2a2+z2b2=1.

Dividing the coordinates by (a,a,b) turns that ellipsoid into the unit sphere. In the scaled coordinates, with o the scaled satellite position and us the scaled direction reduced to a unit vector, the hit times τ solve

τ2+2 (o⋅us) τ+∥o∥2−1=0.

The physical distance along d is t=τ/∥dscaled∥. Intersections with t≤1 m are discarded. Of the remaining forward hits, the nearer one is the ground intercept. A negative discriminant means the ray misses the Earth; a discriminant that is negative only by a numerical graze (greater than −10−10 in the scaled coordinates) is treated as a tangent hit. If both mathematical intersections lie behind the satellite, the direction is labeled as pointing away from the Earth.

The intercept is converted back to geodetic latitude, longitude, and height. Height comes out at the millimeter level of the ellipsoid and is not a terrain elevation. The lab does not apply a terrain model.

## 6.8	Which intercept is reported

Every refined direction is intersected in this way. Directions that miss, or that only intersect behind the satellite, are kept in the candidate table and are not eligible to be the fix.

The reported fix is the eligible intercept with the smallest phase residual ρ. Other eligible intercepts stay in the table. The fix is marked ambiguous in either of two situations:

- the horizontal baselines have rank less than 2, so one direction cosine was never observed; or

- two or more eligible intercepts have phase residuals within 1.0∘ of the best residual, and those directions are separated by at least 1.0∘.

The second test is there so that a single broad basin is not counted as two solutions. Separation below 1.0∘ collapses those directions into one family. Ambiguity means that the wrapped phases do not pick out a unique ground point. The highlighted latitude and longitude are still the lowest residual, and the table says so.

# 7	How the error is scored

## 7.1	Angular error and ground error

After a fix has been chosen, two errors are computed from the simulated truth. Neither one is an input to the estimator.

The angular error is the angle between the true body-frame arrival u and the estimated arrival uest:

Δθang=arccos​clip(u⋅uest, −1, 1).

The ground error is the WGS84 geodesic between the true emitter and the estimated intercept.

## 7.2	Off-nadir sensitivity

The sensitivity curve holds the satellite fixed and walks one antenna azimuth from nadir out to the horizon, in steps of 1∘, including the horizon angle itself. At each off-nadir angle θ it drops a ray, records the ground point, and drops a second ray at θ+Δθ. The default perturbation is Δθ=0.1∘. The plotted ground shift is the geodesic between those two intercepts. If the perturbed ray misses the Earth, that sample is left as a gap.

A short geometric argument explains the shape of the curve. Two rays from the satellite separated by a small angle δθ, in radians, are a perpendicular distance R δθ apart when they reach slant range R. The line of sight meets the local horizontal at the elevation angle ε, so the surface crosses the ray bundle at that angle. The distance along the ground is therefore

Δs≈R δθsinε.

At nadir, ε is near 90∘ and Δs≈R δθ, with R about equal to the altitude. Toward the limb, ε falls toward zero, the same angle error is stretched along the surface, and eventually the perturbed ray misses. The curve itself is the geodesic finite difference. The expression above is the approximation the caption quotes, and on the default orbit it agrees with the geodesic to a fraction of a percent until the ray is near the limb.

A higher orbit has a longer slant range and a smaller Earth disk, so the horizon off-nadir angle shrinks and the ground shift at a given off-nadir angle grows.

## 7.3	Monte Carlo

One Monte Carlo trial repeats the inverse with a new realization of the errors the estimator is allowed to have. The truth scene is computed once, with no phase noise, and then hidden from the estimator.

Errors are applied as follows.

- Channel bias, if its standard deviation is nonzero, is drawn once per antenna and held fixed for the whole run. The pair bias is bj=cj−c0.

- Phase noise is a fresh independent Gaussian sample on each reference pair, on every trial, added before wrapping.

- Attitude knowledge error is added to the roll, pitch, and yaw that the estimator uses. Each axis receives an independent N(0,σa2) sample, in degrees. The true attitude that generated the phase is unchanged.

- Position knowledge error shifts the ECEF origin the estimator uses. In the true VVLH frame,

Δr∼N(0, σp2I3),  rest=rsat+RECEF←VVLH Δr.

The three components are along-track, cross-track, and nadir. The orbital basis is not rebuilt from the perturbed position. If the perturbed origin falls inside the ellipsoid, that trial is skipped.

Each trial inverts the noisy wrapped phases from the perturbed origin and attitude. A trial succeeds when at least one candidate ray meets the Earth. The error of a success is the geodesic from the true emitter to the lowest-residual intercept.

Two summaries are reported, each as a median, a 95th percentile, and a root-mean-square

RMS=1ni​ei2.

The unambiguous summary uses only the successes that were not flagged ambiguous. The other summary uses every success, so a trial that picked the wrong alias is included. The fraction of trials that returned a fix, and the fraction of all trials whose fix was still ambiguous, are reported separately. A trial that failed to hit the Earth is not counted as ambiguous.

# 8	A numerical example

The numbers in this section are the lab’s defaults, evaluated in the same functions the interface calls. Frequency is 2.2 GHz, so λ=0.136269 m and λ/2=0.068135 m.

## 8.1	Array example

The default array is a 3-L with L=0.8 m. Both arms are 5.871 λ. The sidebar direction α=35∘, θ=40∘ has

u=(0.526541, 0.368688, 0.766044).

The two unwrapped phases are 1112.824∘ and 779.208∘, which is 3.09118 cycles on the X arm and 2.16447 cycles on the Y arm. After wrapping they are 32.824∘ and 59.208∘. One noise draw at 1∘ RMS with seed 42 moves those wrapped phases to 33.129∘ and 58.168∘.

Shortening both arms to λ/2 at the same direction gives unwrapped phases 94.777∘ and 66.364∘. Both already lie in (−π,π), so wrapping changes nothing.

## 8.2	Satellite example

The default spacecraft is at geodetic 20∘N, 100∘W, altitude 500 km, inclination 53∘, ascending, with zero roll, pitch, and yaw. Its ECEF position is

rsat=(−1 122 754, −6 367 454, 2 338 707) m.

The geocentric latitude of that same vector is 19.886∘. The circular-orbit heading is 39.790∘ east of north, and the argument of latitude is 25.208∘.

Geocentric nadir from this satellite meets the ellipsoid at 20.009∘N, 100∘W, which is 998 m from the geodetic subpoint. At geodetic 45∘N and the same altitude the corresponding separation is 1.557 km.

The default emitter is 25∘N, 95∘W. From the nadir-pointing satellite the arrival is

α=2.233∘,  θ=54.059∘,  R=930.129 km,

and the body-frame unit vector is (0.809003, 0.031543, 0.586958). On the 0.8 m L the unwrapped phases are 1709.796∘ and 66.665∘. Along the X arm, near this direction, the phase changes by 21.63∘ for each degree of off-nadir. The search grid is spaced by Δu=0.035 in direction cosine. At 54∘ off-nadir that spacing is about 3.4∘ of angle and about 74∘ of phase on the long arm, which is wider than one ambiguity basin. With zero noise the true direction fits the wrapped phases exactly, and so do many other directions. The twelve candidates retained from the 2∘ grid are other basins. Ten of them meet the Earth, all with a numerical phase residual of zero, and the lowest-listed intercept is about 741 km and 52.6∘ away from the emitter. That is the wrapping ambiguity the geolocation tab is warning about, made larger by a grid that cannot see every basin.

Replacing the 0.8 m arms with λ/2=0.068135 m leaves a single Earth intercept. The unwrapped phases of the same geometry are 145.620∘ and 5.678∘, the phase residual of the fix is numerically zero, and the geodesic error is numerically zero.

## 8.3	Sensitivity example

Along body +X at the default satellite, the Earth horizon is 67.906∘ off-nadir. A 0.1∘ increase in off-nadir moves the ground point by:

| Off-nadir | Slant range | Elevation | Ground shift | Approximation R δθ/sinε |
| --- | --- | --- | --- | --- |
| 0∘ | 500.0 km | 89.88∘ | 872.7 m | 872.7 m |
| 20∘ | 535.2 km | 68.26∘ | 1006 m | 1006 m |
| 40∘ | 672.9 km | 46.01∘ | 1635 m | 1632 m |

The approximation uses δθ=0.1∘ in radians. It matches the geodesic at nadir and is still within 0.2% at 40∘ off-nadir.

# 9	Scope of the result

The lab is a simulator of this phase geometry. A reported latitude is the ellipsoid intercept of the lowest-residual direction the search managed to keep. On a short baseline, with the emitter in view and the attitude and position known, that intercept reproduces the emitter. On a long baseline the same residual can belong to several directions, the grid can miss the true basin, and the highlighted fix can sit far from the transmitter even when the phase noise is zero. The phase-residual column measures agreement with the wrapped phases. The geodesic column, which is available only because the simulation knows the truth, measures agreement with the emitter.